Lab 4 Post-Lab, Dustevich & Wallace – C&C Industries

- Predicted Link-Margin Plot:

Figure 1 – Predicted Link-Margin for Dipole and PCB links

- Predicted Link-Margin Plot with Experimental Data Included:

Figure 2 – Experimental Data Overlay on Predicted

- The difference between the expected value and the measured value is that the measured is lower than expected. This likely occurs due to a culmination of multiple factors, such as interference from the ground, especially metal used in concrete, people walking between the transmitter and receiver, and interference from WiFi. Since our operating frequency matches that of WiFi, this likely provides the greatest source of error. The change in this interference between points is most likely due to multipath effects, where the signal may reflect off surfaces differently based on where the receiver was. Most notably, at about 120 m, there was a bridge that would likely stifle some reflection and decrease margin.

- When the transmitter antenna was turned to “point at” the receiver the expected outcome was that the connection would drop, as the receiver would be in the cone of silence of the transmitter. This means the only signal capable of being received would be the reflected signal, which was not strong enough to establish a connection.

- The calculated link margin was found to be –37.77 dB using MATLAB. This means this configuration would not be feasible. To correct this issue, we assume the system is still in development, so the on-orbit component can still be changed. Given this, the design suggestions that could make this link feasible would be to increase the power transmitted as well as the gain of the dipole by perhaps substituting this component for one with a more directionally pointed transmiter/receiver. Both would increase the cost of the project as increasing the transmitter power not only takes more electricity but could also require infrastructure to be upgraded to accommodate this change. Changing parts when the initial design parts are already acquired means an increase in cost as well. Lastly, changing the directionality of a transmitter/receiver requires greater pointing accuracy, once again meaning infrastructure and control programs could require an upgrade.